# F5 BIG-IP HSL over HTTPS — Integration Guide

This guide walks through configuring F5 BIG-IP to mirror API traffic to an external security engine using High-Speed Logging (HSL) over HTTPS.

## Step 1: Configure the Integration Profile

- In the API Security UI, navigate to Settings > Integrations > Traffic Sources.

- Click Add Integration and select the F5 tile.

- Enter a name for the integration profile.

- Select HSL as the method.

- Select the remote engine to send traffic to.

- Download and extract the provided zip file — it contains the iRule TCL script.

## Step 2: Create the Engine Pool

Navigate to: Local Traffic > Pools > Pool List > Create

- Name the pool (e.g., noname-security-engine-https-pool).

- Set health monitor to http (HTTPS probes may fail even though traffic flows over TLS).

- Set node to the engine's hostname or IP address.

- Set service port to 443.

## Step 3: Create the Engine Virtual Server

Navigate to: Local Traffic > Virtual Servers > Virtual Server List > Create

- Name the virtual server (e.g., noname-security-engine-https-vs).

- Set destination IP to an unused internal IP (not an F5 Self IP).

- Set service port to 80 (internal relay only — not externally exposed).

- Set HTTP Profile (Client) to http.

- Set SSL Profile (Server) to serverssl to encrypt traffic to the engine.

- Set Source Address Translation to Auto Map.

- Set Default Pool to the engine pool created in Step 2.

## Step 4: Create the HSL Pool

Navigate to: Local Traffic > Pools > Pool List > Create

- Name the pool (e.g., noname-security-hsl-https).

- Set health monitor to tcp_half_open.

- Set the node address to the IP of the virtual server created in Step 3.

- Set service port to 80 (matching the VS port from Step 3).

## Step 5: Upload the HSL iRule

Navigate to: Local Traffic > iRules > iRule List > Create

- Name the iRule (e.g., noname-hsl-https-logger).

- Paste the full contents of the TCL script downloaded in Step 1.

- Verify key settings in the script: pool name, engine hostname, payload max size, source key, and engine URL.

## Step 6: Attach the iRule to the Application Virtual Server

- Navigate to Local Traffic > Virtual Servers > Virtual Server List.

- Click on your application virtual server.

- Select the Resources tab.

- In the iRules section, click Manage.

- Move the iRule to the Enabled list and click Finished.

Do not attach the iRule to the engine virtual server — this will cause a traffic loop.

## Step 7: Save the Configuration

- SSH into the F5 device.

- Run: tmsh save sys config

- Run this after every configuration change to persist settings across reboots.

## Step 8: (Optional) Configure Traffic Exclusions

- Navigate to Local Traffic > iRules > iRule List and select your iRule.

- Find the line: set nn_filtered_hosts {}

- Add hostnames or IPs to exclude, e.g., set nn_filtered_hosts {"domain.com" "127.0.0.1"}

- Click Update, then save config via SSH.

# Verification

- Navigate to Local Traffic > Pools and confirm both pools show a green/healthy status.

- Navigate to Local Traffic > iRules > select your iRule > Statistics tab.

- Generate traffic to the application virtual server and confirm Total Executions increases.

- Check the HSL pool statistics to confirm Total Requests is increasing.

- Log into the API Security UI and confirm traffic begins appearing within a few minutes.