F5 BIG-IP HSL over HTTPS

Noname / Akamai API Security Integration

Lab Environment: mcropsey-juiceshop-f5.eastus.cloudapp.azure.com

# Overview

This document captures the complete configuration steps for integrating F5 BIG-IP with the Noname (Akamai) API Security platform using High-Speed Logging (HSL) over HTTPS. The iRule mirrors API traffic from your application virtual server to the Noname engine in encrypted asynchronous batches.

Traffic flow:

juiceshop_vs  →  HSL iRule  →  noname-security-hsl-https pool

→  noname-security-engine-https-vs  →  noname-security-engine-https-pool

→  michaels-lab.nonamesec.com:443 (TLS encrypted)

# Environment Details

| Setting | Value |
| --- | --- |
| F5 Hostname | mcropsey-juiceshop-f5.eastus.cloudapp.azure.com |
| F5 Version | BIG-IP 21.0.0.1 Build 0.0.13 |
| Engine Hostname | michaels-lab.nonamesec.com |
| Engine IP | 98.88.158.72 |
| Engine Port | 443 |
| Application VS | juiceshop_vs (10.20.4.10, port 80) |
| Engine VS IP | 10.20.4.100 |
| Engine VS Port | 80 |
| Source Key (UUID) | 04f72d92-81f0-4c5f-8142-7f2b60142ac9 |
| iRule Version | 5.0.0 |

# Step 1: Configure Integration Profile (API Security UI)

- Settings > Integrations > Traffic Sources In the API Security UI, navigate to

- Add Integration and select the F5 tile Click

- Enter a name for the integration profile

- HSL Select

- Select the remote engine to send traffic to (michaels-lab.nonamesec.com)

- Download and extract the Zip file — this contains send-to-noname.tcl

# Step 2: Create Pool for the Engine

Navigate to: Local Traffic > Pools > Pool List > Create

| Setting | Value |
| --- | --- |
| Name | noname-security-engine-https-pool |
| Health Monitor | http (Note: https/https_443 failed; http works for this engine) |
| Node selection | Node List |
| Address | michaels-lab.nonamesec.com (98.88.158.72) |
| Service Port | 443 |

⚠️ Note: The health monitor was changed from 'https' to 'http' because the engine endpoint does not respond to F5's HTTPS health probe format, but traffic still flows over TLS via the serverssl profile on the Virtual Server.

# Step 3: Create Virtual Server for the Engine Pool

Navigate to: Local Traffic > Virtual Servers > Virtual Server List > Create

| Setting | Value |
| --- | --- |
| Name | noname-security-engine-https-vs |
| Source Address | 0.0.0.0/0 |
| Destination IP | 10.20.4.100 (unused IP, NOT an F5 Self IP) |
| Service Port | 80 (internal relay only — not externally exposed) |
| HTTP Profile (Client) | http |
| HTTP Profile (Server) | (Use Client Profile) |
| SSL Profile (Server) | serverssl (encrypts traffic to engine) |
| Source Address Translation | Auto Map |
| Default Pool | noname-security-engine-https-pool |

⚠️ Note: Port 80 is safe here because this VS is internal/loopback only — no external traffic reaches it. Encryption to the engine is handled by the serverssl SSL Profile (Server), so all data sent to Noname is fully TLS encrypted.

# Step 4: Create Pool for the Virtual Server

Navigate to: Local Traffic > Pools > Pool List > Create

| Setting | Value |
| --- | --- |
| Name | noname-security-hsl-https |
| Health Monitor | tcp_half_open |
| Node Name | noname-security-engine-https-vs |
| Address | 10.20.4.100 (IP of VS created in Step 3) |
| Service Port | 80 (port of VS created in Step 3) |

⚠️ Note: This pool is the HSL target. The iRule sends traffic here, which forwards to the engine VS, which then sends to the engine over HTTPS.

# Step 5: Upload HSL iRule

Navigate to: Local Traffic > iRules > iRule List > Create

| Setting | Value |
| --- | --- |
| Name | noname-hsl-https-logger |
| Definition | Paste full contents of send-to-noname.tcl |
| iRule Version | 5.0.0 (already includes Step 6.2 fix) |

Key settings pre-configured in the TCL file:

- nn_pool = noname-security-hsl-https

- nn_engine_hostname = michaels-lab.nonamesec.com

- nn_payload_max_size = 131072 (128KB)

- nn_base64_source_key = 04f72d92-81f0-4c5f-8142-7f2b60142ac9

- nn_engine_url = /engine?message-format=base64

⚠️ Note: The Step 6.2 fix (for iRule versions before 4.0.0) is already applied in this iRule — it uses 'set nn_pool noname-security-hsl-https' before calling HSL::open. No manual fix required.

# Step 6: Attach iRule to Application Virtual Server

Attach the iRule to juiceshop_vs ONLY — do NOT attach to noname-security-engine-https-vs.

- Navigate to Local Traffic > Virtual Servers > Virtual Server List

- Click on juiceshop_vs

- Select the Resources tab

- In the iRules section, click Manage

- Move noname-hsl-https-logger to the Active/Enabled list

- Click Finished

⚠️ Note: Do NOT attach the iRule to noname-security-engine-https-vs — this would cause a traffic loop.

# Step 7: Save Configuration

F5 BIG-IP 21 does not have a simple GUI Save button. Use SSH:

ssh admin@mcropsey-juiceshop-f5.eastus.cloudapp.azure.com

tmsh save sys config

Always run this after any configuration changes to persist the config across reboots.

# Step 8: Create Exclusion List (Optional)

To exclude specific hosts from being mirrored to Noname, edit the iRule:

- Navigate to Local Traffic > iRules > iRule List

- Select noname-hsl-https-logger

- Find line: set nn_filtered_hosts {}

- Update it with hosts to exclude, for example:

set nn_filtered_hosts {"domain.com" "127.0.0.1"}

- Click Update

- Save config via SSH: tmsh save sys config

# Verification

## Check Pool Status

Navigate to Local Traffic > Pools > Pool List and confirm:

- noname-security-hsl-https — green circle ✓

- noname-security-engine-https-pool — green circle ✓

## Check Traffic is Flowing

- Local Traffic > iRules > iRule List > select noname-hsl-https-logger > Statistics tab

- Generate traffic to juiceshop_vs

- Confirm Total Executions count increases

- Navigate to Local Traffic > Pools > Pool List > noname-security-hsl-https > Statistics

- Confirm Total Requests count increases

## Check API Security UI

Log into the API Security UI and navigate to your integration. Traffic from juiceshop should begin appearing within a few minutes of the iRule being attached and traffic flowing.

# Troubleshooting

## Traffic Not Appearing in UI

- Verify iRule is attached to juiceshop_vs (Resources tab)

- Check iRule Statistics — executions should increase with traffic

- Check pool noname-security-hsl-https is green/healthy

- Verify nn_engine_hostname in iRule matches your actual engine domain

set nn_engine_hostname "michaels-lab.nonamesec.com"

- Check firewall — port 443 must be open from F5 to 98.88.158.72

## Pool Showing Red/Down

- Try changing health monitor from https to http

- Test connectivity from F5 bash: curl -vk https://michaels-lab.nonamesec.com

- Test DNS: ping michaels-lab.nonamesec.com

# Known Limitations

- Payload limit is 128KB (nn_payload_max_size). Larger payloads are skipped.

- Integration cannot read encrypted traffic when F5 uses SSL Passthrough.

- If another iRule uses HTTP::respond before this iRule, this iRule will not execute.

- If another iRule uses HTTP::collect before this iRule, this iRule will not execute.

- F5 versions 14.1.4.6, 14.1.5.x with HTTP chunking may have issues.