F5 BIG-IP HSL over HTTPS

Noname / Akamai API Security

Step-by-Step Deployment Guide

Environment: mcropsey-lab

| Setting | Value |
| --- | --- |
| VAmPI (Direct) | http://3.21.112.55:5000/ |
| VAmPI via F5 VIP | http://3.149.175.85/ |
| Swagger UI | http://3.21.112.55:5000/ui/ |
| F5 TMUI | https://16.58.193.153/ |
| F5 Self IPs | 10.0.5.10 / 10.0.6.10 / 10.0.7.20 |
| Engine VS IP | 10.0.6.100 (internal subnet .100) |
| Engine VS Port | 80 |
| Engine Hostname | mcropsey-lab.nonamesec.com |
| Engine Pool | noname-security-engine-https-pool |
| HSL Pool | noname-security-hsl-https |
| Engine VS Name | noname-security-engine-https-vs |
| iRule Name | noname-hsl-https-logger |

# Overview & Architecture

This guide configures F5 BIG-IP to mirror API traffic to the Noname/Akamai API Security engine using High-Speed Logging (HSL) over HTTPS. The iRule captures full HTTP request/response pairs — including headers and bodies — and ships them asynchronously to the engine without affecting application latency.

Key Constraints

SSL Mode: This integration requires SSL Offloading or SSL Bridging on the F5. It does NOT work with SSL Passthrough because encrypted payloads cannot be read by the iRule. The iRule sends data in asynchronous batches — application traffic is never blocked or delayed.

## Traffic Flow

- Client sends API request to F5 VIP (3.149.175.85).

- F5 forwards request to backend pool (VAmPI at 3.21.112.55:5000).

- iRule (noname-hsl-https-logger) captures request + response data.

- iRule sends captured data via HSL/TCP to pool: noname-security-hsl-https.

- That pool targets the Engine VS (10.0.6.100:80) — an internal HTTP relay.

- Engine VS forwards traffic over HTTPS (port 443) to noname-security-engine-https-pool.

- Engine pool resolves to mcropsey-lab.nonamesec.com:443 — the Noname engine.

## Objects Created in This Guide

| Setting | Value |
| --- | --- |
| noname-security-engine-https-pool | Pool pointing to Noname engine at mcropsey-lab.nonamesec.com:443 |
| noname-security-engine-https-vs | Virtual Server (10.0.6.100:80) that relays HSL data to engine pool over HTTPS |
| noname-security-hsl-https | Pool pointing to Engine VS (10.0.6.100:80) — target of the iRule HSL send |
| noname-hsl-https-logger | The iRule — captures traffic and sends to noname-security-hsl-https pool |

Step 1  Create a Pool for the Engine (HTTPS)

Purpose

This pool is the final destination for your traffic data. It holds the address of the Noname engine (mcropsey-lab.nonamesec.com) and uses HTTPS health checks to confirm the engine is reachable. The Engine VS created in Step 2 will use this pool as its default backend.

- Log in to F5 TMUI: https://16.58.193.153/

- Navigate to Local Traffic > Pools > Pool List and click Create.

- Configure the pool with the following settings:

| Setting | Value |
| --- | --- |
| Name | noname-security-engine-https-pool |
| Health Monitor | https (move from Available to Active) |
| Node Name | nn-engine |
| FQDN | mcropsey-lab.nonamesec.com ( if using remote engine you have to use the ip of the remote engine ) |
| Service Port | 443 |

- Under New Members, select FQDN (not New Node), then enter the values above.

- Click Add, then click Finished.

Why FQDN?

Use FQDN (not IP address) for the node. This allows the F5 to automatically follow DNS changes if the engine IP ever changes. The FQDN option does not support non-default route domains.

### Required: Create a Custom HTTPS Health Monitor

The Noname engine always runs behind an AWS Application Load Balancer (ALB). The ALB requires a correct Host header to route requests correctly. The default F5 https monitor sends the IP address as the Host header, which causes the ALB to return 404, marking all pool members down. A custom monitor is required that sends the correct hostname. Run the following from bash on the F5:

tmsh create ltm monitor https noname-https-monitor defaults-from https \ send "GET / HTTP/1.1\r\nHost: 10.0.8.100\r\nConnection: close\r\n\r\n" \ recv "HTTP" tmsh modify ltm pool noname-security-engine-https-pool monitor noname-https-monitor tmsh save sys config

Step 2  Create a Virtual Server for the Engine Pool

Purpose

The Engine Virtual Server acts as an internal HTTPS relay. The iRule cannot send directly over HTTPS, so it sends plain HTTP to this VS, which then wraps the connection in TLS and forwards it to the engine pool. This is why the VS listens on HTTP (port 80) but uses serverssl on the server side.

- Navigate to Local Traffic > Virtual Servers > Virtual Server List and click Create.

- Configure the Virtual Server with the following settings:

| Setting | Value |
| --- | --- |
| Name | noname-security-engine-https-vs |
| Source Address | 0.0.0.0/0 |
| Destination Address/Mask | 10.0.6.100 (internal subnet — NOT a Self IP) |
| Service Port | 80 |
| HTTP Profile (Client) | http |
| HTTP Profile (Server) | (Use Client Profile) |
| SSL Profile (Server) | serverssl |
| Source Address Translation | Auto Map |

Warning

The Destination IP 10.0.6.100 must NOT be one of the F5 Self IPs. Your Self IPs are: 10.0.5.10, 10.0.6.10, 10.0.7.20. 10.0.6.100 is confirmed available in the 10.0.6.x subnet.

- Click Finished. The virtual server is created.

- Navigate to the virtual server's Resources tab.

- Under Load Balancing, set Default Pool to: noname-security-engine-https-pool

- Click Update.

Record for Next Step

Record these values — they are needed in Step 3: Destination IP: 10.0.6.100 Service Port: 80

Step 3  Create a Pool for the Virtual Server (HSL Target)

Purpose

This pool is what the iRule actually sends data to via HSL::send. It points to the Engine VS (10.0.6.100:80) created in Step 2. The F5 uses a tcp_half_open health monitor here because the VS is an internal relay — a full HTTP check is not needed at this layer.

- Navigate to Local Traffic > Pools > Pool List and click Create.

- Configure the pool with the following settings:

| Setting | Value |
| --- | --- |
| Name | noname-security-hsl-https |
| Health Monitor | tcp_half_open (move from Available to Active) |
| Node Name | noname-security-engine-https-vs |
| Address | 10.0.6.100 (IP of the Engine VS from Step 2) |
| Service Port | 80 (Port of the Engine VS from Step 2) |

- Under New Members, select New Node, then enter the values above.

- Click Add, then click Finished.

Critical: Pool Name Must Match iRule

The pool name noname-security-hsl-https is hardcoded in your iRule (send-to-noname.tcl). It appears in the CLIENT_ACCEPTED event: set nn_pool noname-security-hsl-https Do not rename this pool or the iRule will fail to open its HSL connection.

Step 4  Upload the HSL iRule

Purpose

The iRule is the core of the integration. It hooks into F5's HTTP event pipeline to capture request and response data (method, URL, headers, body, status, timestamps) and ships it asynchronously to the Noname engine via the HSL pool. Your iRule is pre-configured with your specific engine hostname, source key, and source version.

- Navigate to Local Traffic > iRules > iRule List and click Create.

- Set Name to:  noname-hsl-https-logger

- In the Definition field, paste the full contents of your send-to-noname.tcl file.

- Click Finished.

Your iRule Configuration

Key values already embedded in your iRule (no changes needed): Engine hostname: mcropsey-lab.nonamesec.com HSL pool target: noname-security-hsl-https Engine URL path: /engine?message-format=base64 Max payload size: 131072 bytes (128 KB) Source key: e3f5e227-bb59-460c-9a58-5daa6dbdf95e iRule version: 7.1.0

Older iRule Workaround (Pre-4.0.0)

If you are running an iRule version older than 4.0.0, you must apply a workaround. In the iRule editor, find this line: set nn_hsl [HSL::open -proto TCP -pool noname-security-hsl-https] And replace it with these two lines: set nn_pool noname-security-hsl-https set nn_hsl [HSL::open -proto TCP -pool $nn_pool] Then click Update. Your iRule at version 7.1.0 likely does not need this fix.

Step 5  Attach the iRule to Your Application Virtual Server

Purpose

This step activates the integration. Attaching the iRule to your application's Virtual Server (the VIP that serves VAmPI at 3.149.175.85) tells F5 to run the iRule on every HTTP transaction that passes through it. No application traffic is blocked or modified — only mirrored.

- Navigate to Local Traffic > Virtual Servers > Virtual Server List.

- Click on your application Virtual Server (the one serving traffic to 3.149.175.85).

- Click the Resources tab.

- In the iRules section, click Manage.

- Move noname-hsl-https-logger from Available to Enabled.

- Click Finished.

Warning

Only attach this iRule to Virtual Servers that have HTTP/HTTPS profiles assigned. Do not attach to TCP-only or UDP virtual servers — the HTTP:: commands will error. If you have an existing iRule with HTTP::respond, ensure the Noname iRule runs BEFORE it, otherwise the Noname iRule will not execute on responses.

AS3 Alternative

If you use AS3 for deployments, add the following to the relevant virtual server JSON instead: "iRules": [ { "bigip": "/Common/noname-hsl-https-logger" } ]

Step 6  Create an Exclusion List (Optional)

Purpose

By default the iRule sends traffic from ALL hosts through the Virtual Server to Noname. If there are hosts you want to exclude (e.g. health check endpoints, internal monitors), you can list them in the iRule and their traffic will be silently skipped.

- Navigate to Local Traffic > iRules > iRule List.

- Click noname-hsl-https-logger to open the editor.

- Find this line near the top of the iRule:

set nn_filtered_hosts {}

- Replace the empty braces with your excluded hosts, for example:

set nn_filtered_hosts {"10.0.6.10" "internal-monitor.local"}

- Click Update.

Exclusion List Format Rules

Do not include http:// or https:// prefixes. Do not include port numbers. Wrap each entry in double quotes and separate with a space. Subdomains are NOT automatically included — list each one explicitly. Example: domain.com does NOT cover sub.domain.com.

Step 7  Verify the Integration

Purpose

Before declaring the integration complete, verify that data is flowing through each layer. Work top-down: confirm the iRule executes, then that the HSL pool receives requests, then that the engine pool is healthy.

### 7a — Verify the iRule Is Executing

- Navigate to Local Traffic > iRules > iRule List.

- Click noname-hsl-https-logger.

- Click the Statistics tab.

- Generate traffic to your VIP (e.g. curl http://3.149.175.85/).

- Confirm the Total Executions counter increases.

### 7b — Verify the HSL Pool Is Receiving Requests

- Navigate to Local Traffic > Pools > Pool List.

- Click noname-security-hsl-https.

- Click the Statistics tab.

- Generate traffic and confirm Total Requests increases.

### 7c — Verify the Engine Pool Is Healthy

- Navigate to Local Traffic > Pools > Pool List.

- Click noname-security-engine-https-pool.

- Confirm the pool member (mcropsey-lab.nonamesec.com:443) shows as Available (green).

- If it shows as Unknown, go to Health Monitors, activate https, and click Update. Refresh the page.

### 7d — Fix Host Header if Traffic Still Not Appearing

Some load balancers require a correct Host header to route the request. If traffic is not appearing in the Noname UI after the above checks pass, update the host header in the iRule:

- Navigate to Local Traffic > iRules > iRule List.

- Click noname-hsl-https-logger.

- Find the variable nn_1777922589718_engine_hostname and confirm it reads:

set static::nn_1777922589718_engine_hostname "mcropsey-lab.nonamesec.com"

- If it is incorrect, update it and click Update.

# Known Limitations

- Payload size is capped at 128 KB per transaction. Oversized payloads are sent without body. Adjustable via nn_payload_max_size in the iRule but larger values impact F5 performance.

- SSL Passthrough is not supported — the iRule cannot inspect encrypted traffic.

- If an existing iRule uses HTTP::respond, the Noname iRule must run before it or it will not capture the response.

- If an existing iRule uses HTTP::collect, the Noname iRule must run after it.

- If an existing iRule uses HTTP::redirect, also attach the Noname iRule to the redirect destination Virtual Server.

- F5 versions 14.1.4.6, 14.1.5.x through 14.1.5.6 with HTTP chunking enabled may not work correctly due to an F5 bug.

- The integration may count against your F5 license bandwidth limits.

# Updating the iRule

When a new version of the integration is available, download the new Zip from the Akamai API Security UI and replace only the iRule content — all pools and virtual servers remain unchanged.

- Download and extract the new Zip from Akamai API Security > Settings > Integrations.

- Navigate to Local Traffic > iRules > iRule List.

- Click noname-hsl-https-logger.

- Replace the entire Definition with the contents of the new send-to-noname.tcl.

- Click Update.

# Uninstalling

To remove the integration without deleting the application Virtual Server:

- Navigate to Local Traffic > Virtual Servers > Virtual Server List.

- Click your application Virtual Server.

- Click the Resources tab.

- In the iRules section, click Manage.

- Move noname-hsl-https-logger from Enabled back to Available.

- Click Finished.

- (Optional) Delete the following objects if no longer needed:

| Setting | Value |
| --- | --- |
| iRule | noname-hsl-https-logger |
| Pool | noname-security-hsl-https |
| VS | noname-security-engine-https-vs |
| Pool | noname-security-engine-https-pool |
| Node | nn-engine (mcropsey-lab.nonamesec.com) |

- Delete the integration profile in Akamai API Security > Settings > Integrations.

© 2026 Akamai Technologies — mcropsey-lab deployment